3GPP TSG-WG2 Meeting #174	S2-2602039

# Malta, MT, 13 – 18 April 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5687 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to URSP rule removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI19 |  |  |  |  |  | Date: |  |  | 2026-03-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - It is described as PCF-initated UE policy association termination in step 4 in the figure 4.16.13.2-1. Actually, it shall be PCF-initiated UE policy association modification. - Step 4 referes to steps 7, 8 and 9 of procedure UE policy association modification initiated by the PCF in clause 4.16.2.2 to remove UE policies. Step 7 describes (V-)PCF triggers UE Configuration Update Procedure to sends the UE policy container. So the interactions between H-PCF and V-PCF is missed in current descriptions. - It shall referr to AMF-initiated UE policy assocation termination in step 7. The word "Termination" is missed in current figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PCF-initiated UE policy association modificatin is corrected in the figure - Steps 3 – 9 of procedure UE policy assocatio modification initiated by the PCF is referred. - PCF-initiated UE policy association termination is corrected in the figure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect procedure. UE policy stored at UE can’t be removed which leads to insistent between UE and network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

4.16.13.2	PCF-initiated UE Policy Association Termination

Figure 4.16.13.2-1: PCF-initiated UE Policy Association Termination

This procedure concerns both roaming and non-roaming scenarios.

In the non-roaming case, the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the H-PCF interacts with the V-PCF to request removing Policy Association.

The PCF is subscribed to notification of changes in Data Set "Policy Data" for a UE Policy Association ID.

1.	The Policy data is removed, either the Data Set "Policy Data" or the Data Subset "UE context policy control".

2.	The UDR sends the Nudr_DM_Notify_Request (Notification correlation Id, Policy Data, SUPI, UE Context Policy Control data, updated data) including the SUPI, the Data Set Identifier, the Data Subset Identifier and the Updated Data including empty "Policy Data" or empty "UE context policy control".

3.	The PCF sends the Nudr_DM_Notify_Response to confirm reception and the result to UDR.

4.	The PCF may initiate the removal of the UE Policies in the UE by invoking steps 3 - 9 of procedure UE Policy Association Modification initiated by the PCF in clause 4.16.12.2. Alternatively, following the same steps from clause 4.16.12.2 the PCF may decide to update the UE Policies in the UE based on a default profile.

5.	The PCF may notify the AMF of the removal of the UE Policy Association via Npcf_UEPolicyControl_UpdateNotify service operation. Alternatively, the PCF may decide to maintain the Policy Association if a default profile is applied, in this case steps 5, 6 and 7 are not executed.

In the non-roaming case, the PCF unsubscribes to analytics from NWDAF if any.

6.	The AMF acknowledges the operation.

7.	Steps 2-5 in clause 4.16.13.1 AMF-initiated UE Policy Association Termination are performed to remove the UE Policy Association for this UE and the subscription to Policy Control Request Triggers for that UE Policy Association.

************** End of changes **************